Assessment

Course -Btech CSE  Robotics  and AI

Discipline - SOET

Program - Nand -to-Tetris

Name - Ajitesh Singh Gusain

Faculty – Dr.  Ayyala kishore Ajay Kumar

Sem - 3

Course code - ETCCNT301

Lab 1 - Question 1: Implementation of Elementary Logic Gates Using Universal NAND Gates

1. Objective & Theoretical Background

The objective of this section is to construct basic logic gates (NOT, AND, and OR) using exclusively NAND gates. In digital circuit design, the NAND gate is a universal gate, meaning any Boolean logic function can be implemented using combinations of NAND gates alone.

2. Gate Implementations & Analysis

A. NOT Gate (Inverter)

- Concept: A NOT gate flips a single binary input (0→1 and 1→0). A NAND gate outputs 0 only when both inputs are 1. By tying both input pins of a NAND gate together to the same input signal, the gate acts as an inverter.

- Boolean Expression: out=in=∈NAND∈

- HDL Code:

Code snippet

CHIP Not {

IN in;

OUT out;

PARTS:

Nand(a=in, b=in, out=out);

}

- Logic Diagram Setup:

Y = (A·A)̅

= A̅

So only 1 NAND gate is required.

Simulation Verification: Ran Not.tst in the Hardware Simulator. The script passed all steps (=0→out=1, =1→out=0).

Truth  table

| Input (in) | NAND Input a | NAND Input b | Output (NOT(in)) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 |

B. AND Gate

- Concept: An AND gate outputs 1 only when both inputs a and b are 1. A standard NAND gate outputs 0 when both inputs are 1. Passing the output of a NAND gate through a NAND-based inverter flips the result back to positive AND logic.

- Boolean Expression: out=a⋅b=a⋅b=aNANDbNANDaNANDb

- HDL Code:

Code snippet

CHIP And {

IN a, b;

OUT out;

PARTS:

Nand(a=a, b=b, out=nandOut);

Nand(a=nandOut, b=nandOut, out=out);

}

Logic Diagram Setup:

Y = [ (A·B)̅ · (A·B)̅ ]̅

Y = A·B

- Simulation Verification:

Truth table

| Input a | Input b | Stage 1 (nandOut) | Stage 2 / Final Output (out) |
| --- | --- | --- | --- |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

C. OR Gate

- Concept: An OR gate outputs 1 if at least one input is 1. According to De Morgan’s Law (out=a+b=a⋅b), an OR operation is built by inverting inputs a and b individually using NAND gates, then passing those inverted signals into a final NAND gate.

- Boolean Expression: out=a+b=a⋅b=aNANDaNANDbNANDb

- HDL Code:

Code snippet

CHIP Or {

IN a, b;

OUT out;

PARTS:

Nand(a=a, b=a, out=notA);

Nand(a=b, b=b, out=notB);

Nand(a=notA, b=notB, out=out);}

Logic Diagram Setup:

- Simulation Verification:

Truth  table

| Input a | Input b | Stage 1 (notA) | Stage 2 (notB) | Stage 3 / Final Output (out) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 |