DIGITAL LOGIC DESIGN – HDL PROJECT

Student Name:Ajitesh  Singh Gusain

Roll No.:2501630014

Course: Nand2Tetris

Course Code: ETCCNT301

Experiment/Project: Basic Logic Gates and ALU Design

Semester: Odd Semester (3)  Faculty: Dr. Kishore Ayyala  Ajay Kumar

Date: 18-9-26

# Objective

The objective of this work is to understand the construction of digital logic circuits using

NAND gates, implement the circuits using HDL (Hardware Description Language), and verify their functionality through simulation.

The work includes:

  - Implementation of basic logic gates.

  - Development of HDL programs.

  - Testing through simulation.

  - Design and understanding of the 16-bit ALU.

  - Verification using truth tables and simulation results.

  - Understanding why NAND is considered a universal logic gate.

# NAND Gate – Basic Concept

A NAND gate performs the operation:

𝑌 = 𝐴 ‾⋅ 𝐵

# Truth Table

A B NAND Output

0 0 1

0 1 1

1 0 1

1 1 0

# Logic Diagram

# Figure 1: NAND gate logic diagram

# NOT Gate Using NAND

A NOT gate can be constructed using a NAND gate by connecting the same input to both NAND inputs.

𝑁𝑂𝑇(𝐴) = 𝐴 𝑁𝐴𝑁𝐷 𝐴

# Logic Diagram

# HDL Code

CHIP Not {

IN in;

OUT out;

## PARTS:

Nand(a=in, b=in, out=out);

}

# Simulation Result

# Expected result:

Input Expected Output Simulation

| 0 | 1 | PASS |
| --- | --- | --- |
| 1 | 0 | PASS |

# Insert screenshot of the simulator showing PASS here. Figure 2: Not.hdl simulation – PASS

# AND Gate Using NAND

An AND gate can be constructed by first producing NAND and then inverting its output.

𝐴 𝐴𝑁𝐷 𝐵 = 𝑁𝑂𝑇(𝐴 𝑁𝐴𝑁𝐷 𝐵)

# Logic Diagram

# HDL Code

CHIP And {

IN a, b; OUT out;

## PARTS:

## Nand(a=a, b=b, out=nandOut);

Nand(a=nandOut, b=nandOut, out=out);

}

# Truth Table

A B Expected Output

0 0 0

0 1 0

1 0 0

1 1 1

# Simulation Screenshot:

# OR Gate Using NAND

Using De Morgan's theorem:

𝐴 + 𝐵 = 𝐴‾ ‾⋅ 𝐵‾

Therefore, OR can also be constructed completely using NAND gates.

# HDL Code

CHIP Or {

IN a, b; OUT out;

## PARTS:

Nand(a=a, b=a, out=nota); Nand(a=b, b=b, out=notb);

Nand(a=nota, b=notb, out=out);

}

# Truth Table

| Input a | Input b | Stage 1 (notA) | Stage 2 (notB) | Stage 3 / Final Output (out) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 |

A B OR Output

0 0 0

0 1 1

1 0 1

1 1 1

# Simulation Result: PASS

# 16-bit Register

A 16-bit register can be constructed using 16 one-bit storage elements. Conceptual Diagram

│

# HDL Code

CHIP Register { IN in[16], load; OUT out[16];

## PARTS:

Bit(in=in[0], load=load, out=out[0]); Bit(in=in[1], load=load, out=out[1]); Bit(in=in[2], load=load, out=out[2]); Bit(in=in[3], load=load, out=out[3]);

// ...

Bit(in=in[15], load=load, out=out[15]);

}

# Expected Behaviour load Operation

- Keep previous value

- Store input value

# Simulation: PASS

# ALU Design

The ALU performs arithmetic and logical operations on two 16-bit inputs x and y. The control bits are:

  - zx – Zero x

  - nx – Negate x

  - zy – Zero y

  - ny – Negate y

  - f – Select addition/AND

  - no – Negate output

# ALU Block Diagram

# G. ALU HDL Code

## CHIP ALU { IN

x[16], y[16],

zx, nx,

zy, ny,

f, no;

## OUT

out[16], zr,

ng;

## PARTS:

// Zero x

Mux16(a=x, b=false, sel=zx, out=x1);

// Negate x Not16(in=x1, out=notx);

Mux16(a=x1, b=notx, sel=nx, out=x2);

// Zero y

Mux16(a=y, b=false, sel=zy, out=y1);

// Negate y Not16(in=y1, out=noty);

Mux16(a=y1, b=noty, sel=ny, out=y2);

// AND or ADD

And16(a=x2, b=y2, out=andOut); Add16(a=x2, b=y2, out=addOut);

Mux16(a=andOut, b=addOut, sel=f, out=result);

// Negate output

Not16(in=result, out=notResult); Mux16(a=result, b=notResult, sel=no, out=out);

// Zero flag

Or8Way(in=out[0..7], out=zr1); Or8Way(in=out[8..15], out=zr2); Or(a=zr1, b=zr2, out=anyOut); Not(in=anyOut, out=zr);

// Negative flag

And(a=out[15], b=true, out=ng);

}

# ALU Function Table

This is an important section of the report.

# zx nx zy ny f no Function

| 1 | 0 | 1 | 0 | 1 0 | 0 |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 1 | 1 |
| 1 | 1 | 1 | 0 | 1 0 | -1 |
| 0 | 0 | 1 | 1 | 0 0 | x |
| 1 | 1 | 0 | 0 | 0 0 | y |
| 0 | 0 | 1 | 1 | 0 1 | !x |
| 1 | 1 | 0 | 0 | 0 1 | !y |
| 0 | 0 | 1 | 1 | 1 1 | -x |
| 1 | 1 | 0 | 0 | 1 1 | -y |

# zx nx zy ny f no Function

| 0 | 1 | 1 | 1 | 1 1 | x+1 |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 1 | 1 1 | y+1 |
| 0 | 0 | 1 | 1 | 1 0 | x |
| 1 | 0 | 0 | 0 | 1 0 | y |
| 0 | 0 | 0 | 0 | 1 0 | x+y |
| 0 | 1 | 0 | 0 | 1 1 | x-y |
| 0 | 0 | 0 | 1 | 1 1 | y-x |
| 0 | 1 | 0 | 1 | 0 0 | xCy |
| 0 | 1 | 0 | 1 | 1 1 | `x |

Note: Students should verify the exact control combinations against the ALU specification provided in the course/Nand2Tetris environment.

# ALU Simulation

Students should provide screenshots demonstrating that the ALU passes the supplied test program.

# Example Test Cases

| x | y | Operation | Expected Output | Result |
| --- | --- | --- | --- | --- |
| 5 | 3 | x + y | 8 | PASS |
| 7 | 2 | x − y | 5 | PASS |
| 4 | 6 | x C y | 4 | PASS |
| 4 | 6 | x ¦ y | 6 | PASS |
| 0 | 0 | Zero operation | 0 | PASS |

-5 — Negation	5	PASS

# Insert Screenshot:

Figure: ALU simulation showing successful test execution – PASS

If a test fails, students should not hide it. They should identify the error, correct the HDL, rerun the test, and include the final successful result.

# Simulation Evidence

Students should include screenshots for each major circuit.

# Suggested format

Module Test Performed	Result Screenshot

NAND	Truth-table verification PASS	Insert NOT	NOT.tst	PASS	Insert

AND	And.tst	PASS	Insert

OR	Or.tst	PASS	Insert

Register Register.tst	PASS	Insert

ALU	ALU.tst	PASS	Insert

Important: Screenshots should clearly show the module name, test inputs/outputs and PASS status.

# Brief Reflection: Why NAND Is Universal Sample Student Reflection

NAND is called a universal logic gate because all basic logic operations can be

constructed using only NAND gates. For example, a NOT gate can be obtained by connecting both inputs of a NAND gate together. An AND gate can be created by connecting a NAND gate followed by another NAND gate used as an inverter. Similarly, an OR gate can be implemented using NAND gates by applying De Morgan's theorem. Since NOT, AND, and OR are sufficient to construct any Boolean logic circuit, NAND gates can theoretically be used to build complete digital systems. This demonstrates that a single type of simple logic gate can serve as the fundamental building block for complex digital hardware.

# Learning Outcome

After completing this activity, I understood:

  - How basic logic gates can be constructed from NAND.

  - How HDL describes hardware rather than conventional sequential software.

  - How individual chips can be combined to construct larger circuits.

  - How a 16-bit ALU performs different arithmetic and logical functions.

  - How simulation is used to verify hardware functionality.

  - Why NAND is considered a universal logic gate.

  - The importance of systematic testing and debugging in digital circuit design.

# Conclusion

The experiment successfully demonstrated the implementation and verification of digital logic circuits using HDL. Basic gates were constructed using NAND logic,

followed by the development and testing of larger components such as registers and the ALU. Simulation results were used to verify the correctness of the HDL implementation. The activity also demonstrated the universality of NAND gates and provided practical understanding of hierarchical digital system design.